Time to Winterize Your Cabin

As temperatures have dropped, you want to think about how you ‘left’ your cabin on your last trip, or more accurately if you properly winterized in preparation for the Winter. To help you, here are some tips that we have picked up from long term Ponderosa residents.

Should I turn off water each time I leave the cabin? Anyone that does not live in their residence ‘full time’, and those full time residents that leave for extended periods of time for vacations, etc. should always turn off their water as a precaution, even in the Summer months. It only takes a few minutes and if a leak/break occurs, the only amount of water that can potentially leak out will be what is actually in the lines at the time. This simple precaution could save thousands of gallons of water and the homeowner a costly bill.

When Should I Winterize? It is recommended that you ‘winterize’ your cabin beginning in the fall, typically around Thanksgiving Day. By then the temperatures typically drop below freezing each night.

How to Winterize: While the ‘set up’ in each cabin can differ, the following is a simplified version of the procedure.

1) Go to the Water Heater and turn the settings dial to ‘vacation’. Older Water Heaters may not have a ‘vacation’ setting, if so turn the dial to the lowest setting. By placing the water heater on the low setting, it will use a minimal amount of propane or electricity to keep the water warm and hence prevent it from freezing in the tank. If you intend to shut off your water heater completely, then you should drain it completely.

2) Go into the ‘sub area’ (underneath the cabin) and find the water supply line where it enters the foundation. This will ‘typically’ be a copper or galvanized pipe line coming out of the ground with a shut off valve very close to this point. The ‘shut off’ valve may be either a round handle not unlike the handle to your hose bib, or it can also be a ‘directional’ handle. If it is a directional handle, turn it until the handle is at a 90 degree angle to the incoming water line.  In either case, make sure the valve completely shuts off water to your cabin. You can test this by opening a valve inside your cabin (i.e. kitchen sink); no water should come out of the faucet if the shut off valve is properly closed.

3) Once the water is turned off, find the lowest water faucet (preferably on the exterior) and open the faucet, and leave it open. If there is no exterior faucet, open whatever faucet is at the ‘lowest’ elevation point in the cabin.

4) Now, go back through the cabin and open every water faucet, shower/tub faucet, and flush every toilet. As an additional safety measure, make sure to partially open the shower head ‘diverter’ valve, if so equipped. You goal is to eliminate as much water from the pipes in your system as possible.

5) Next, pour antifreeze into each and every drain in the cabin including shower drains, sink drains, and toilets (both the bowl and tank).  The exact amount of antifreeze will depend on the type of antifreeze you use - read the directions on the container carefully. Some use regular auto antifreeze and others use antifreeze designed for RV plumbing. You might also find some ideas through internet research.

6) Lastly, close the door to your Dishwasher (if applicable) and turn on the Dishwasher for just a few seconds. This will activate the solenoid in the Dishwasher allowing what little water is in the line serving the Dishwasher to drain and prevent the plastic solenoid from freezing and breaking. You may need to do the same thing if you have a washing machine and also set the ice maker in your freezer to the “on” position as well as the water dispenser on the refrigerator, if you have one.

7) As an added precaution, some of us leave our heater (assuming you have one) set to a low setting of about 50 degrees.  This will keep the interior of your cabin warm enough to prevent interior pipes from freezing, but most heaters only work if the electric power is on. In the event of an extended power outage, this extra precaution will be nullified.

Conclusion: Even though you properly shut down and drained your cabin, it is virtually impossible to evacuate all of the water from the system. There will always be an amount of water still in the pipes. The amount of water depends on if the plumbing system was properly ‘sloped’ when it was installed. When we experience freezing conditions, the remaining water in the system can still freeze and result in broken pipes. If you or someone else has not been to your cabin recently, we have very strong words of caution. Do not turn on the water and leave the cabin without looking and listening for breaks!

We hope you find these tips helpful, and if you have any questions about how to properly winterize your cabin, then we suggest you contact one of the long term or full time residents who can offer specific advice for your situation.  The following people may be able to help you if you have questions:

Bill Dolmovic		559-542-8000

Richard Thomas	559-542-2970

Kit Korf		559-542-2504

Jimmy Parminter	559-542-2524